2017 Advanced Computer Networks Homework 1

M063040050 曾國維

Part 1: Web Browsing (DNS, TCP)

- Find the first DNS request packet sent by the client. (Request for cse.nsysu.edu.tw)

  - Examine the Ethernet

    - What is the Ethernet address of the source and destination?

Ans:	Source: 54:ab:3a:5b:2e:44

Destination: 00:1c:73:00:09:99

    - What is the content of the type field in the Ethernet frame?

Ans:	IPv4(0x0800)

  - Examine the Internet Protocol

    - What is the IP address of the source and destination?

Ans:	Source: 140.117.220.89

Destination: 140.117.205.1

    - What is the header length? What is the total packet length?

Ans:	Header length: 20bytes

Packet length: 62bytes

    - Identify the protocol type field. What is the number and type of the protocol in the payload?

Ans:	UDP(17)

  - Examine the User Datagram Protocol

    - Identify the client ephemeral port number and the server well-known port number.

Ans:	client ephemeral port number:55461

server well-known port number:17

    - What type of application layer protocol is in the payload?

Ans:由UDP port53可知，協定為DNS

  - Examine the Domain Name System (query)

    - What field indicates whether the message is a query or a response?

Ans:由Flags第1個bit為0可以知道message is a query

    - What is the query transaction ID?

Ans:	0xec6d = 60525(10)

    - Identify the fields that carry the type and class of the query.

And:	Type:A

Class:IN

- Find the DNS response packet which is response to the DNS request packet from the above question.

  - Examine the Ethernet

    - What is the Ethernet address of the source and destination?

Ans:	Source: 00:1c:73:1a:2c:ac

Destination: 54:ab:3a:5b:2e:44

    - What is the content of the type field in the Ethernet frame?

Ans:IPv4(0x0800)

  - Examine the Internet Protocol & Domain Name System (response)

    - What is the IP address of the source and destination?

Ans:	Source:140.117.205.1

Destionation:140.117.220.89

    - What is the header length? What is the total packet length? Is it longer than the query?

Ans:	Header length:20bytes

Packet length:115bytes，Query的長度是包含在packet當中，所以packet length較長

    - How many answers are provided in the response message? Compare the answers and their time-to-live values.

Ans:	RR type不同，A為查詢主機位址紀錄，NS為定義有管轄權的名稱伺服器，TTL分別為:300,37,37

- Find the first TCP packet sent by client. (The destination IP address is response from above question.)

  - Examine the Transmission Control Protocol

    - What are the ephemeral port number used by the client and the well-known port number used by the server?

Ans: ephemeral port number:51292

well-known port number:80

    - What is the length of the TCP segment?

Ans:	0

    - What is the initial sequence number for the segments from the client to the server?

Ans:	0

    - What is the initial window size?

Ans:	64240

    - What is the maximum segment size?

Ans:	1460bytes

    - Find the hex character that contains the SYN flag bit

Ans:0x002

Part 2: Probing the Internet (ICMP, PING, Traceroute)

- Ping Captured.

  - Find the first ICMP Echo Request packet.

    - First, examine the Internet Protocol. What is the Time-to-Live?

Ans:	128

    - Next examine the Internet Control Message Protocol. What is the ICMP message type?

Ans:	8(echo ping request)

  - Find the first ICMP Echo Reply packet.

Ans:	TTL:58

ICMP message type:0(echo ping reply)

- Traceroute Captured.

  - Find the first ICMP Echo Request packet.

    - Examine the Internet Protocol. What are the source and destination addresses?

Ans:	source:140.117.220.89

Destination:8.8.8.8

    - What are the protocol type and the Time-to-Live in the IP packet?

Ans:	ICMP,TTL:1

    - Next, examine the Internet Control Message Protocol. What is the ICMP message type? What are the message identifier and sequence number?

Ans:	(BE) identifier:256, sequence number:68

(LE) identifier:1, sequence number:17408

  - Find an ICMP Time-to-live exceeded packet.

    - Examine the Internet Protocol. What are the source and destination addresses?

Ans:	source:140.117.221.252

Destionation:140.117.220.89

    - Next, examine the Internet Control Message Protocol. What is the ICMP message type?

Ans:8(echo ping request)

Part 3 Measuring Network Bandwidth

使用-w 2000 將window size設成2000K使得buffer減小，Transfer rate下降

使用-b 512G 將bandwidth 設為512G bits/sec ，Transfer rate上升，但buffer size不變讓UDP有較高的封包遺失率。